USING PRO DESKTOP TO DESIGN YOUR CASE

As part of your Design Folio you will need to produce a CAD drawing of your product case.

Draw your own case by following these steps.

- Open PTC Pro Desktop

- Top left of the screen SELECT –New-Design-OK

- Maximise design window.

Draw your profile by:

- Select the ‘free form’ tool found below the circle and rectangle tools on the right hand side tool bar.

- Click in the middle of the drawing page and draw your shape. This will make the base of your case.

Make it 3-D by:

- The shape you have drawn should be highlighted red.

- Click the ‘Extrude Profile’ button. Grab the yellow handle and adjust the height of case to your liking. Then grab the green handle to reduce the size of the top.

Rounding or Chamfering the edges:

- Select the ‘select edge’ tool on the right side tool bar.

- Click on the edge of your case you want to round. Remember, to select more than one edge at a time, hold down the shift key.

- Check your selected edges of your case are red.

- Click on the round button or the chamfer button at the top of the tool bar.

- A window will appear. Type a distance, between 5 and 10mm, or adjust with the handle on the drawing.

Drawing Buttons

- Click the face tool on the right side tool bar. Select the top of your case, this face should now go red.

- Right click on the face and select ‘New Sketch’.

- Select a circle or rectangle tool on the right hand side tool bar and draw your buttons on the top face of your case.

- Click ‘Extrude button’ grab the yellow handle and pull buttons to the height you require. To make holes ‘Subtract material’ grab handle and drag into your case.

Save your work

- Select File-Save as-Type in ‘case’. Click OK.

- Select Tools-‘Split Screen’

- Print a copy for your design folder.

Rendering

- Select NEW-Album

- Right click on the grey area of the middle of the screen. Select ‘New Image’ then select your ‘case’ file. OK.

- Click on the case so that turns red.

- On the left side of the screen look through the materials and click one you like. Right click on the material and select ‘Apply Materials’ then click on the ‘update button’ (Traffic Light above middle tool bar)

- Save Album and print in colour.